Readme file-

- Do the connections as mentioned in below table.

| # | Source | Target |
| --- | --- | --- |
|  | Wifi-Module | Relay |
| 1 | Any I/O pin (I have used PIN 4) | S (SIgnal) |
| 2 | 5V | (+)ve |
| 3 | GND | (-)ve |
|  |  | LDR |
| 4 | 3V3 | 1st leg |
| 5 | A/O PIN | 2nd leg |
|  |  | Resistor |
| 6 | A/O PIN | 1st leg |
| 7 | GND | 2nd leg |
|  | Relay | Bulb Holder |
| 8 | COM | 1st pin |
|  |  | 2 Pin Plug |
| 9 | NO | 1st pin |
|  | Bulb Holder | 2 Pin Plug |
| 10 | 2nd Pin | 2nd Pin |

- The circuit should look like below.

- I have set the threshold value to switch on the bulb at 350 for this project.

Input = When senor value > threshold value

Output= Light is off

- Input = When senor value < threshold value

Output= Light is on using the bolt IoT module code

- Input = LDR not working or the coding is not working or any other breakdown due to which we are unable to get input value from bolt API(In this project, I have changed input function name to something that doesn’t exist, so that the code is unable to get input).

Output= Light is on using the predictions done by Machine Learning coding (used a sample data input file to create ML code).